Foundation

- Concrete 32 mpa type 50

- Footings 24” x 8” minimum with steel rebar

- 9’ Basement Wall’s

# Framing

- Built up Spruce Beams or Engineered beams as specified on plan

- Engineered floor truss system as specified on plan

- Sub floor 23/32” T&G O.S.B. – glued and screwed

- Exterior framing – 2” x 6” KD Spruce Studs at 16” on centre

- Interior framing – Spruce Studs at 16” on centre or to match floor system

- 2 x 4 Spruce studs, frost walls 24” on centre

- 9 foot main floor ceiling.

Garage and Driveway (as per plan)

- Driveway full width of garage front

- Framing 2” x 6” KD Spruce Studs at 16” on centre

- Fully insulated, dry walled and fire taped.

- Overhead door Insulated, pre-finished, raised panel – white enamel

- Garage Overhead Door, Opener complete with two remotes

- Keyless entry

- Garage Floor Drain

# Exterior Doors and Windows

- Front door – Fibreglass embossed, insulated, or metal

- Maintenance free double-glazed, pre-finished white vinyl picture,

(As per plan) complete Low E with screens

- Bars on front elevation

- Double pain windows

- Basement windows sized for egress/ window wells to meet Egress 30 x 30

# Plumbing

- Waterlines Home Run System

- Minimum 141 L (50 US Gallon) hot water  tank

- Roughed In Double Plumbing

- Kitchen sink Double Stainless Steel (Under mount) with single lever chrome faucet

- Hose bibs as per Plan

- Gas line to Garage, Kitchen and Future deck.

- Vanity sinks white with single lever chrome faucet

- Toilets white china

- Tubs white fibreglass 1 piece tub/shower combo/ master bath set up with tub option

- Roughed in dishwasher

- Shut off valve on each tap

- Water line to fridge

- Garage hot and cold taps

- Floor drain laundry

# Electrical

- 100 amp circuit panel – minimum 30-60 circuits

- Telephone – 5 outlets (as per plan)

- Cable outlets – 4 outlets

- Standard lighting package as per plan

- Roughed in central vacuum system with 2 outlets for future use

- Smoke detectors wired to house electrical system/ all rooms as per code

- Decorah Plugs and Switches/ all bedrooms GFI as per code

- Under LED cabinet lighting

## Heating

- Fireplace – Standard (as per plan). Electrical or upgrade to gas

Fireplace with ceramic tile surround and stain grade mantel

- Furnace – high-efficiency gas

- Furnace sized to meet heat loss calculations

- Humidifier on furnace

Insulation and Drywall

- Exterior Wall – R22 fibreglass insulation and poly vapour barrier

- Ceiling insulation – R50 loose fill or Batt insulation with poly vapour

Barrier on top floor

- Frost walls full height 2 x 4 framing with full height R20 insulation and poly vapour barrier

- Spray Foam insulation on cantilevers and rim joists

- ½” drywall on exterior & interior walls in developed areas

- Soft line bull nose corner or 45 degree corner

- Know down ceiling in all areas.

Cabinets

- Professionally Crafted Maple Raised panel or  Shaker Doors

- Quartz Countertops

- Soft close hinges on drawers

- Crown Moulding and Valence

Interior Finishing

- Passage & closet doors – paint grade raised panel style doors.

- Door hardware – antique nickel or chrome.

- Baseboards 4 ¼” painted MDF

- Door & Window Casings – 3 ½” painted MDF

- Shelving –Built in MDF

- Carpet – 100% nylon cut pile or Berber with 7# underlay.

- Hardwood, Carpet, Tile on Main Floor and bathrooms

- Paint – 1 wall color and 1 trim color

- 1 coat of primer and 2 coats of latex finish on walls in all areas

- Interior doors and trim to be painted with latex semi-gloss finish

- Framed mirrors with Italian wood moldings

- Maple railing with Iron rods.

Exterior Finishing

- Pre-finished vinyl siding on all sides as per plan

- Parging applied as required

- Exterior trims and accent boards on front elevations as noted on plans

- 30 year asphalt shingles

- 5” pre-finished aluminium eaves trough

- Exterior ledge stone

Miscellaneous

- Exterior Steps: Pre-cast/poured concrete as per plan

- Sidewalks are broom finished

- Appliances (Stainless Steel, white washer/ dryer)

*Specifications are subject to change without notice.  Error and omissions exempted.  See sales consultant for details.

In keeping with a continually changing market and our pursuit to provide you with the best possible features at a fair price, Dynasty Builders reserves the right to change or add equivalent products to these typical features in our homes.