Morinville Silver Specification Highlights

Foundation



· Concrete 32 mpa type 50




· Footings 24” x 8” minimum with steel rebar
· 8’ Basement Wall

Framing

· Built-up spruce beams or engineered beams, as specified on plan

· Engineered floor truss system, as specified on plan

· Sub floor 23/32” T&G O.S.B. – glued and screwed

· Exterior framing – 2” x 6” KD spruce studs at 16” on centre

· Interior framing – spruce studs at 16” on centre 

· Basement Frost Walls 2 x 4 spruce studs @ 24” on centre
Garage and Driveway (as per plan)

· Driveway full width of garage front 
· Garage Framing 2” x 4” KD spruce studs at 16” on centre
· Garage fully insulated, drywalled and fire taped
· Overhead Garage Door insulated, pre-finished, raised panel – white enamel complete with two remotes and keyless entry
Exterior Doors and Windows

· Front door – insulated fibreglass.
· Patio doors with screen in nook/kitchen
· Window grills on front elevation, as per plans
· Sliders windows double pain

· Basement windows, as per plans, sized for egress

Plumbing

· Waterlines home run system

· Hot Water Tank 141 L (50 US Gallon) 
· Roughed In Double Plumbing in basement
· Kitchen Sink double stainless steel with single lever chrome faucet 

· Exterior Hose Bibs, as per plan
· Vanity Sinks white with single lever chrome faucet

· Toilets white china

· Tubs white fibreglass 1 piece tub/shower combo
· Shut off valve on each tap 
Electrical

· 100 amp circuit panel – minimum 30-60 circuits 

· Telephone – 1outlet in kitchen 
· Cable outlets – 1 in great room and 1 in master bedroom
· Standard ceiling lighting package 

· Roughed in central vacuum system with 2 outlets for future use

· Smoke detectors wired to house electrical system
· Decora Plugs and Switches

Heating

· Gas Fireplace 
· Furnace – high-efficiency gas

· Furnace sized to meet heat loss calculations 

· Regular plate type humidifier on furnace
· HRV System
Insulation and Drywall

· Exterior Wall – R20 fibreglass insulation and poly vapour barrier

· Ceiling insulation – R50 loose fill or bat insulation with poly vapour

         barrier on top floor

· Frost walls full height 2 x 4 framing with full height R20 insulation and poly vapour barrier 

· ½” drywall on exterior & interior walls in developed areas 
· 90 degree corner beads
· Regular Stippling on ceilings
Cabinets

· Maple shaker doors cabinets
· Quartz or Granite Countertops in kitchen and bathrooms
Interior Finishing

· Passage & closet doors – paint grade raised panel style doors
· Door hardware –  antique chrome
· Baseboards 3 ¼” painted MDF

· Door & Window Casings – 2 ½” painted MDF

· Shelving – Durable pre-finished wire shelving white in color
· Built in Master Closet in MDF
· Carpet – 100% nylon cut pile or Berber with 7 lb. underlay
· Laminate flooring on main floor, carpet on stairs, bedrooms and Tile in upper level bathrooms
· Paint – 1 wall color and 1 trim color; 1 primer and 2 finish coats of latex paint on all house walls 

· Interior doors and trim to be painted with latex semi-gloss finish
· Framed mirrors with Italian wood moldings

Exterior Finishing

· Pre-finished vinyl siding on all sides as per plan

· Parging applied as required

· Exterior trims and accent boards on front elevations as noted on plans 

· 30 year asphalt shingles

· 5” pre-finished aluminium eaves trough 

· Exterior cultured ledge stone

Miscellaneous

· Exterior Steps are PWF wood frame
· Kitchen appliances with stainless steel fronts included (Fridge, Stove, Dishwasher and over the range Microwave Hood Fan)
*Specifications are subject to change without notice.  Error and omissions exempted.  See sales consultant for details.

[image: image1.png]



In keeping with a continually changing market and our pursuit to provide you with the best possible features at a fair price, Dynasty Builders reserves the right to change or add equivalent products to these typical features in our homes.

